Job Shop Scheduling Problems

- n jobs, m machines

- Each job follows a predetermined route

(a sequence of machines)

Problem:

- Operation (i,j) : Processing of job j on machine i

- Processing time pij

- Assume  that jobs do not recirculate

- Minimize Cmax

Example 1: 3 jobs, 4 machines

| Jobs | Machine Sequence | Processing Times |
| --- | --- | --- |
| 1 | 1,2,3 | p11=10, p21=8, p31=4 |
| 2 | 2,1,4,3 | p22=8, p12=3, p42=5, p32=6 |
| 3 | 1,2,4 | p13=4, p23=7, p43=3 |

Disjunctive Programming

- A set of constraints is conjunctive, if each constraint in the set must be satisfied

- A set of constraints is disjunctive, if at least one of the  constraints in the set must be satisfied

- Any integer program can be written as a disjunctive program

Disjunctive Programming Formulation

(Job Shop Problem, Minimize Makespan)

Variables:

tij = start time of job j on machine i,      for all operations (i,j)

Min Cmax

s.t

Cmax   tij + pij				for all (i,j)  N

tkj  tij + pij 				for all (i,j)  (k,j)   A

tij  tik + pik   or  tik  tij + pij  	for all (i,j) and (i,k)  N

tij   0 		 			for all (i,j)  N

Example 1: revisited, 3 jobs, 4 machines

| Jobs | Machine Sequence | Processing Times |
| --- | --- | --- |
| 1 | 1,2,3 | p11=10, p21=8, p31=4 |
| 2 | 2,1,4,3 | p22=8, p12=3, p42=5, p32=6 |
| 3 | 1,2,4 | p13=4, p23=7, p43=3 |

Min Cmax

s.t

Cmax   t31 + p31

Cmax   t32 + p32

Cmax   t33 + p43

t21  t11 + p11

t31  t21 + p21

t12  t22 + p22

t42  t12 + p12

t32  t42 + p42

t23  t13 + p13

t43   t23 + p23

for i=1,2,3,4:

ti1    ti2 + pi2	or 	ti2   ti1  + pi1

ti1    ti3 + pi3	or 	ti3   ti1  + pi1

ti2    ti3 + pi3	or 	ti3   ti2 + pi2

tij   0, 		for all (i,j)  N

IP Formulation:

Variables:

tij = start time of job j on machine i,      for i = 1,2,3 and j =1,2,3

xijk =   1, if  job j precedes job k on machine i,

0, otherwise      					for i = 1,2,3,4

j =1,2,3 and  j < k

Min Cmax

s.t

Cmax   t31 + p31

Cmax   t32 + p32

Cmax   t33 + p43

t21  t11 + p11

t31  t21 + p21

t12  t22 + p22

t42  t12 + p12

t32  t42 + p42

t23  t13 + p13

t43   t23 + p23

ti1 + pi1    ti2 + M(1-xi12)		for i=1,2,3,4

ti2 + pi2    ti1 + M xi12

ti1 + pi1   ti3 + M(1-xi13)

ti3 + pi3    ti1 + M xi13

ti2 + pi2    ti3 + M(1-xi23)

ti3 + pi3   ti2 + M xi23

tij   0, 		for i=1,2,3,4 and j=1,2,3

xijk  {0,1} 	for i = 1,2,3,4 	j =1,2,3 and  j < k

Disjunctive Graph Representation:

- Directed graph G, nodes N, arc sets A and B, G=(N,A,B)

- There exists a node for each operation (i,j)

- Conjunctive arcs A represent routes of the jobs

Arc (i,j)      (k,j)  denotes that operation (i,j) precedes (k,j)

(defined for two operations of the same job)

- Disjunctive arcs B represent sequence of jobs on a machine

Arc (i,j)      (i,k)  denotes that operation (i,j) precedes (i,k)

Arc (i,j)      (i,k)  denotes that operation (i,k) precedes (i,j)

Arcs in both directions exist, only one of the two is selected in a feasible schedule

(defined for two operations on the same machine)

Length of an arc is the processing time of the operation  from which the arc originates    (i,j)      (k,j)

pij

- Add a source node U and a sink node V

- Connect U to the first operation of each job by a conjunctive arc (going out of U) of length 0

- Connect V to the last operation of each job by a conjunctive arc (going into V)

- A  feasible schedule corresponds to a subgraph S such that

- S contains all the  conjunctive arcs A

- For each pair of disjunctive arcs between the same nodes,

exactly one arc is contained in S

- S contains no directed cycle

Example 1: revisited, 3 jobs, 4 machines

| Jobs | Machine Sequence | Processing Times |
| --- | --- | --- |
| 1 | 1,2,3 | p11=10, p21=8, p31=4 |
| 2 | 2,1,4,3 | p22=8, p12=3, p42=5, p32=6 |
| 3 | 1,2,4 | p13=4, p23=7, p43=3 |

First, add the conjunctive arcs (set A)

U

V

2,2

1,2

4,2

3,2

4,3

1,1

2,1

3,1

1,3

2,3

Now, add the disjunctive arcs (set B) (dashed arcs)

U

V

2,2

1,2

4,2

3,2

4,3

1,1

2,1

3,1

1,3

2,3

0

0

0

p43

p23

- Why does this subgraph not give a feasible schedule?

V

2,2

1,2

4,2

3,2

4,3

1,1

2,1

3,1

1,3

2,3

0

0

0

U

an orientation coresponds to a feasible schedule if and only if the subgraph contains no directed cycles

Makespan of a schedule is the critical path.

- How can we minimize the makespan?

Choose the arcs so as to minimize the length of the critical path.

Active Schedules:

Two types of modification of a feasible schedule to obtain another feasible schedule

- A left shift:  move an operation left to start it earlier

- A left jump:  move an operation left into an idle slot to start it earlier

100

200

MC 4

MC 1

MC 2

310

1

3

3

3

2

2

1

1

MC 3

2

2

Left shift

MC 1

MC 2

300

1

3

3

2

1

MC 3

2

1

2

MC 4

3

2

Left jump

- A feasible schedule is active if it cannot be modified by a left shift or a jump to complete an operation earlier without completing another operation later

200

MC 4

MC 1

MC 2

300

1

3

3

2

2

1

1

MC 3

2

2

3

100

280

Active schedule

Branch and Bound for Disjunctive Programs:

- There exists an optimal solution that is an active schedule

- We can restrict our search to only active schedules

Generating All Active Schedules

- At every level of the search tree one operation is fixed

- At a given level, operations that are already fixed make up a partial schedule

- No operation is considered until all its predecessors are scheduled

- If all the predecessors of an operation are scheduled, then the operation is said to be available

- Given  a partial schedule, all Available Operations can be found easily

Notation:

AO = set of all operations whose predecessors have already been scheduled (Available Operations)

(denoted by  in the textbook)

rij = earliest starting time of operation (i,j) in AO

- Given a partial schedule and  an available operation (i,j), how do we calculate rij ?

Algorithm OFA

Step 1. (Initialization)

AO = {first operation of each job}

rij = 0 	for all (i,j) in AO

Step 2. (Machine Selection)

Calculate 	t(AO) = min(i,j) in AO{ rij + pij}

and let  i* be the machine corresponding to the operation that minimizes rij + pij

Step 3. (Branching)

For each operation (i*,j) on machine i* such that ri*j  < t(AO),

- Construct a branch by fixing that operation as the next operation on machine i*

- delete the operation from AO

- add the immediate follower of the operation to AO

If AO is empty, stop. Else, return to Step 2.

- The key condition that yields active schedules is the inequality ri*j  < t(AO) in Step 3.

- There cannot be any operation that can be completed before t(AO)

...

(i*,k)

Level k+1

Level k

(i*,j)

(i*,l)

(i*,p)

Create a branch for each available operation on machine i* with earliest start time smaller than t(AO)

- Can you think of a lower bound that can be obtained at a node of the enumeration tree?

Given a feasible schedule represented by a disjunctive graph, how do you find the critical path?

- Finding the critical path is equivalent to finding the longest path from u to v in the given acyclic directed graph

- We calculate a label for each node

- Let rij be the label of node (i,j) corresponding to operation (i,j)

- Let IPij be the set of immediate predecessors of (i,j)

IPij contains all nodes that have an arc originating from itself and ending at node (i,j)

- Start with ru = 0 	(source node has label equal to 0)

- Repeat

- Pick a node (i,j) such that the labels of all nodes in IPij have been calculated

- For every node in IPij calculate {label of the node + length of the arc from this node to (i,j)} and set rij to the maximum of these terms

Until the label of v, the sink node, is calculated

- Length of the longest u-v path equals rv, the label of the sink node

- To find the longest path itself trace back the nodes that gave the maximum term in the label calculations

- The label of node (i,j), rij , equals

- the length of the longest path from the source node to (i,j)

- the earliest possible start time of operation (i,j)

Example:

V

2,2

1,2

4,2

3,2

4,3

1,1

2,1

3,1

1,3

2,3

0

0

0

3

7

6

4

8

10

10

10

10

8

8

6

U

3

5

8

8

3

5

4

- Can you think of a lower bound that can be obtained at a node of the enumeration tree?

LB1 =

- How can you improve this lower bound?

Idea:

- When we calculate the makespan of a partial schedule, we allow several operations to be processed on a machine at the same time

- Pick one of the machines and allow all other machines to process multiple operations simultaneously except the chosen machine.

Calculation of the bound:

- Consider machine i

- Compute rij  for all operations (i,j) on machine i

- Find the longest path from (i,j) to the sink for all (i,j) on machine i; let lij be the length of this path

- Set a due date for (i,j):  dij = LB1 - lij  + pij

- Solve the following single machine problem on machine i:

- Operations (i,j) on machine i are the jobs

- Operation (i,j) has release time rij  and due date dij

- No preemptions allowed

- Minimize Lmax, the maximum lateness

6. LB = LB1 + Lmax

Note 1: The optimal sequence of the single machine problem implies new disjunctive arcs; Add the new arcs to the current subgraph and calculate the makespan on the new (temporary) graph. This makespan is the calculated lower bound

Note 2: The lower bound calculation can be repeated for all machines so that the largest lower bound is kept

Branch and Bound

Example 1: revisited, 3 jobs, 4 machines

- Start with a subgraph consisting of only the conjunctive arcs

- Calculate the makespan of this graph

U

V

2,2

1,2

4,2

3,2

4,3

1,1

2,1

3,1

1,3

2,3

0

0

0

3

7

6

4

8

4

5

8

10

3

Makespan =

- Apply algorithm OFA to branch and generate all nodes at level 1

Step1. Initialize

AO = {(1,1), (2,2), (1,3)}

rij = 0 	for all (i,j) in AO

Step 2. (Machine Selection)

Calculate 	t(AO) = min(i,j) in AO{ rij + pij}

t(AO) = min { 0+10, 0+8, 0+4} = 4

and let  i* be the machine corresponding to the operation that minimizes rij + pij

i* = 1

Step 3. (Branching)

For each operation (i*,j) on machine i* such that ri*j  < t(AO),

- Construct a branch by fixing that operation as the next operation on machine i*

- delete the operation from AO

- add the immediate follower of the operation to AO

Branch on operations (1,1) and (1,3)

No disjunctive  arcs,  LB = 22

Level 0

(1,3)

scheduled first on MC 1

(1,1)

scheduled first on MC 1

Level 1

LB = 28

LB = 28

- If (1,1) is scheduled first, then two disjunctive arcs going out of (1,1) are added to the graph

[next, obtain lower bounds]

- Calculate the makespan on the new graph

U

V

2,2

1,2

4,2

3,2

4,3

1,1

2,1

3,1

1,3

2,3

0

0

0

3

7

6

4

8

4

5

8

10

3

10

10

LB1 = 24

- Improve this lower bound

Calculation of the bound at (1,1):

- Consider machine 1

- Compute r11 , r12, and r13

- Find the longest path from (1,j) to the sink for all (1,j) on machine 1; find l11 , l12, and l13

- Set a due date for (i,j):  dij = LB1 - lij  + pij ,

- Solve the following single machine problem on machine 1:

| Jobs | 1 | 2 | 3 |
| --- | --- | --- | --- |
| p1j | 10 | 3 | 4 |
| r1j |  |  |  |
| d1j |  |  |  |

Minimize Lmax, the maximum lateness

The optimal sequence = 1-2-3 and Lmax = 3

LB = LB1 + Lmax = 24 + 3 = 27

- Repeat the LB calculation for machine 2

The optimal sequence = 2-1-3 and Lmax = 4

LB = LB1 + Lmax = 24 + 4 = 28 		keep this bound!

- Repeat the LB calculation for machines 3 and 4, best LB = 28

- If (1,3) is scheduled first, then two disjunctive arcs going out of (1,3) are added to the graph

V

2,2

1,2

4,2

3,2

4,3

1,1

2,1

3,1

1,3

2,3

0

0

0

3

7

6

4

8

4

5

8

10

3

U

4

4

Makespan = LB1 =

- Best lower bound obtained by solving single machine problems is 28

- Branch from node (1,1) at Level 1; apply algorithm OFA

AO = {(2,1), (2,2), (1,3)}

r22 = 0, r21 = 10, r13 = 10

Calculate 	t(AO) = min(i,j) in AO{ rij + pij}

t(AO) = min { 0+8, 10+8, 10+4} = 8 and i* = 2

Branch on operation (2,2)

No disjunctive  arcs,  LB = 22

Level 0

(1,3)

scheduled first on MC 1

(1,1)

scheduled first on MC 1

Level 1

LB = 28

LB = 28

(1,1)

scheduled first on MC 1

(2,2)

scheduled first on MC 2

LB = 28

Level 2

- Process the currently only node at level 2

- The four disjunctive arcs going out of (1,1) and (2,2) are added to the original graph

V

2,2

1,2

4,2

3,2

4,3

1,1

2,1

3,1

1,3

2,3

0

0

0

3

7

6

4

8

4

5

8

10

3

10

10

8

U

8

LB1 =

Lmax = 0 for all single machine problems, so LB = 28

If all the unprocessed nodes in the branch and bound tree are processed, the following optimal solution with makespan 28 is obtained:

| Machine | Job Sequence |
| --- | --- |
| 1 | 1,3,2 or (1,2,3) |
| 2 | 2,1,3 |
| 3 | 1,2 |
| 4 | 2,3 |